Microprocessor and Computer Architecture Laboratory

UE19CS256

4th Semester, Academic Year 2020-21

Date: 12/02/2021

| Name: Pranav R. Hegde | SRN: PES1UG19CS343 | Section: F |
| --- | --- | --- |

Week#____3_______            Program Number: ___1___

Title of the Program

- ARM Assembly Code for each program

- Output Screen Shot

Problem statement:

1. Write an ALP to add two 64bit numbers loaded from memory and store the result in memory

ARM assembly code:

.text

ldr R0, =P

ldr R1, =Q

ldr R2, [R0], #4

ldr R3, [R0]

ldr R4, [R1], #4

ldr R5, [R1]

adds R7, R3, R5

adc R6, R2, R4

ldr R8, =R

str R6, [R8], #4

str R7, [R8]

ldr R9, =R

ldr R10, [R9, #4]

ldr R11, [R9]

swi 0x11

.data

P:.WORD 0x00000001, 0xa0000002

Q:.WORD 0x00000003, 0xa0000004

R:.WORD

Output:

Microprocessor and Computer Architecture Laboratory

UE19CS256

4th Semester, Academic Year 2020-21

Date: 12/02/2021

| Name: Pranav R. Hegde | SRN: PES1UG19CS343 | Section: F |
| --- | --- | --- |

Week#____3_______            Program Number: ___2___

Title of the Program

- ARM Assembly Code for each program

- Output Screen Shot

Problem statement:

2. Write an ALP to copy n numbers from Memory Location A to Memory Location B.

Code:

.text

ldr R0, =P

ldr R1, =Q

mov R2, #10

L1:

ldr R3, [R0]

str R3, [R1]

sub R2, R2, #1

add R0, R0, #4

add R1, R1, #4

cmp R2, #0

bne L1

swi 0x11

.data

P:.WORD 2, 4, 6, 40, 10, 12, 14, 16, 18, 20

Q:.WORD

Output:

Microprocessor and Computer Architecture Laboratory

UE19CS256

4th Semester, Academic Year 2020-21

Date: 12/02/2021

| Name: Pranav R. Hegde | SRN: PES1UG19CS343 | Section: F |
| --- | --- | --- |

Week#____3_______            Program Number: ___3___

Title of the Program

- ARM Assembly Code for each program

- Output Screen Shot

Problem statement:

3. Write an ALP to find smallest number in an array of n – 32 bit numbers.

Code:

.text

ldr R0,=A

ldr R1,=B

ldr R4,[R0],#4

ldr R3,[R1]

beq L1

sub R3,R3,#1

mov R6,R4 ; Smallest number to be stored in R6

L2:

ldr R4,[R0],#4

cmp R4,R6

movlt R6,R4

sub R3,R3,#1

cmp R3,#0

bne L2

L1:

SWI 0x11

.data

A:.WORD 5,13,21,4,3,19,30,8,16,42

B:.WORD 10

Output:

Microprocessor and Computer Architecture Laboratory

UE19CS256

4th Semester, Academic Year 2020-21

Date: 12/02/2021

| Name: Pranav R. Hegde | SRN: PES1UG19CS343 | Section: F |
| --- | --- | --- |

Week#____3_______            Program Number: ___4a____

Title of the Program

- ARM Assembly Code for each program

- Output Screen Shot

Problem statement:

4a. Write an ALP to count the number of 1’s and 0’s in a given 32 bit number.

Code:

.text

ldr R0,=0b11110010100101001101001100110111

mov R1,#32

mov R5,#0

mov R6,#0

L1:

and R2,R0,#1

cmp R2,#1

addeq R5,R5,#1 ;Number of 1's is stored in R5

addne R6,R6,#1 ;Number of 0's is stored in R6

mov R0,R0,LSR #1

sub R1,R1,#1

cmp R1,#0

bne L1

swi 0x11

.end

Output:

Microprocessor and Computer Architecture Laboratory

UE19CS256

4th Semester, Academic Year 2020-21

Date: 12/02/2021

| Name: Pranav R. Hegde | SRN: PES1UG19CS343 | Section: F |
| --- | --- | --- |

Week#____3_______            Program Number: ___4b___

Title of the Program

- ARM Assembly Code for each program

- Output Screen Shot

Problem statement:

4b. Write an ALP to find the number of zeroes, positive and negative numbers in a given array.

Code:

.text

ldr R1,=A

ldr R2,=B

ldr R3,[R2]

mov R7,#0

mov R8,#0

mov R9,#0

L1:

ldr R4,[R1],#4

cmp R4,#0

addeq R7,R7,#1 ;Number of 0's is stored in R7

addlt R8,R8,#1 ;Number of negative numbers is stored in R8

addgt R9,R9,#1 ;Number of positive numbers is stored in R9

sub R3,R3,#1

cmp R3,#0

bne L1

swi 0x11

.data

A:.WORD -1,-2,-3,0,1,2,0,4,5,6

B:.WORD 10

Output:

Microprocessor and Computer Architecture Laboratory

UE19CS256

4th Semester, Academic Year 2020-21

Date: 12/02/2021

| Name: Pranav R. Hegde | SRN: PES1UG19CS343 | Section: F |
| --- | --- | --- |

Week#____3_______            Program Number: ___5___

Title of the Program

- ARM Assembly Code for each program

- Output Screen Shot

Problem statement:

5. Write an ALP to check whether a given number is present in array using Linear Search (Without SWI 0x02), if found move +1 to R6 and key position to R7 else move -1 to R6 (if number not found).

Code:

.text

ldr R0,=A

mov R1,#10

mov R3,R1

L1:

ldr R2,[R0],#4

cmp R2,#16 ;Element 16 is to be searched in the array

beq L2

subs R1,R1,#1

cmp R1,#0

bne L1

mov R6,#-1

swi 0x11

L2:

mov R6,#1

sub R1,R1,#1

sub R7,R3,R1

swi 0x11

.data

A:.WORD 2,4,6,8,12,20,25,30,40,50

Case1:  Element found

Case2: Element NOT found

Microprocessor and Computer Architecture Laboratory

UE19CS256

4th Semester, Academic Year 2020-21

Date: 12/02/2021

| Name: Pranav R. Hegde | SRN: PES1UG19CS343 | Section: F |
| --- | --- | --- |

Week#____3_______            Program Number: ___6___

Title of the Program

- ARM Assembly Code for each program

- Output Screen Shot

Problem statement:

6. Write an ALP to generate Fibonacci Series and store them in an array.

Code:

.text

ldr R1,=A

mov R2,#0

str R2,[R1]

add R1,R1,#4

mov R3,#1

str R3,[R1]

mov R5,#8 ; 8 fibonacci numbers will be stored after 0 and 1(So,total=10) in the array

L1:

add R4,R2,R3

add R1,R1,#4

str R4,[R1]

mov R2,R3

mov R3,R4

subs R5,R5,#1

mov R4,#0

bne L1

swi 0x11

.data

A:.WORD

Output: